UTC LS-Series Transformer Data

(from OPT-UTC site)

Low Impedance to Grid and Mixing & Matching Transformers

| Type | Application | Primary Impedance | Secondary Impedance | Response 2db (Hz) | Max Level dbm | Rel. Hum | Unbal DC in Pri. | Case No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LS10X | Low impedance mike, pickup or multiple line to PP grids | 50, 125/150, 200/250, 333, 500/600 Ohms | 60,000 Ohms in two sections | 20-20,000 | +17 | -92 db-Q | .5 ma | LS-1 |
| LS12X | Low impedance mike, pickup or multiple line to PP grids | 50, 125/150, 200/250, 333, 500/600 Ohms | 80,000 Ohms overall, split | 20-20,000 | +17 | -92 db-Q | .5 ma | LS-1 |
| LS14X | Low impedance mike, pickup or parallel mixer to grid | 2.5, 5.5, 10, 15, 22, 30, 38, 60 Ohms | 50,000 Ohms | 20-20,000 | +17 | -92 db-Q | .5 ma | LS-1 |

Interstage Transformers

| Type | Application | Primary Impedance | Secondary Impedance | Response 2db (Hz) | Max Level (mw) | Rel. Hum | Unbal DC in Pri. | Case No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LS21 | Plate to PP grids Split Primary & Secondary | 15,000 Ohms | 135,000 Ohms 3:1 Overall | 10-20,000 | 100 | -74 db | 0 ma | LS-1 |
| LS25 | PP plates to PP grids, Med. level split Pri & Secondary | 30,000 Ohms | 50,000 Ohms Turns Raio 1.3:1 Overall | 20-20,000 | 200mw | -74 db | 1 ma | LS-1 |
| LS40 | Plate to PP grids Split Secondary | 15,000 Ohms | 135,000 Ohms 3:1 Overall | 30-18,000 | 100mw | -74db | 8 ma | LS-1 |

Output Transformers to Line & Voice Coil

| Type | Primary Will Match Typical Tubes | Primary Impedance | Secondary Impedance | Response 2db (Hz) | Max Level (W) | Rel. Hum | Unbal DC in Pri. | Case No. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LS55 | PP 300B, 6L6's, 6AS7G, 6080, 7027, 7581, 7355, 786B | 5,000 Ohms P to P 3,000 Ohms P to P | 500,333,250/200,125,50,30,20,15,10,7.5,5, 2.5,1.2 | 7-50,000 | 20 | - | - | LS-2 |
| LS63 | PP triode: 6AS7G, 6080, 6L6, 5881, KT-66, 807, 1614 | 10,000 Ohms P to P 6,000 ohms P to P | 30,20,15,10,7.5,5,2.5,1.2 | 7-50,000 | 20 | - | - | LS-2 |

Case Dimensions

| Case No. | A(+/-.060) | B(+/-.060) | C(+/-.060) | D(+/-.031) | E(+/-.031) | INSERTS |
| --- | --- | --- | --- | --- | --- | --- |
| LS-1 | 2.56 | 2.99 | 3.13 | 2.38 | 2.00 | 4 X #6-32 |
| LS-2 | 3.31 | 3.88 | 4.13 | 2.49 | 3.00 | 4 X #8-32 |